學號：B073040050

姓名：翁昌閔

Homework 5

nMOS I-V Curves and INV DC Transfer Curves

Handout: 2021/06/01

Due: 2021/06/22

- I-V curves for an nMOS transistor with Vdd=1 volt are shown below for both long-channel (large MOS length) and short-channel (small MOS length) transistors. Answer the following questions based on the CIC 0.18um process with Vdd=1.8 volts,

  - Plot the I-V curves for the nMOS width given in the following table.

→ [figure]：

Green: 	short channel 1.8V

Yellow:	short channel 1.6V

Orange:	long channel 1.8V

Blue:	long channel 1.6V

  - Fill in the values of Ids current for the specified Vds and Vgs.

[long_nMOS, Vds=1.8v, Vgs=1.8v]：

[long_nMOS, Vds=1.6v, Vgs=1.8v]：

[short_nMOS, Vds=1.8v, Vgs=1.8v]：

[short_nMOS, Vds=1.6v, Vgs=1.8v]：

  - What are the threshold voltages (in unit of volts) up to the second fractional position? Since threshold voltage Vth depends on Vds and Vgs, give the Vth for Vds=1.8V, Vgs=1.8V. Hint: use finer separation between two neighboring Vgs to observe the Vgs which causes a significant increase of Ids (at Vds=1.8volt  & 1.6volts).

[long_nMOS, Vds=1.8v, Vgs=1.8v]：

[long_nMOS, Vds=1.6v, Vgs=1.8v]：

[short_nMOS, Vds=1.8v, Vgs=1.8v]：

[short_nMOS, Vds=1.6v, Vgs=1.8v]：

  - What are the channel-length modulation coefficients for short-channel nMOS transistors assuming the α-power law model with α=1.5? Hint: you can calculate it based on the current values at different Vds which are in the saturation region. Use the following equation for nMOS with Vds=1.8V and 1.6V, and assume α=1.5 where the threshold voltage has been measured in the previous problem.

| long/short channel | width W (um) | length L (um) | W/L | Vds (V) | Vgs (V) | Ids (uA) | Vt (V) | (α=1.5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| long | 2.5 | 1.8 | 1.39 | 1.8 | 1.8 | 316.4679u | 352.8049m | -0.00659 |
|  | 2.5 | 1.8 | 1.39 | 1.6 | 1.8 | 316.4679u | 352.8049m | -0.00659 |
| short | 0.25 | 0.18 | 1.39 | 1.8 | 1.8 | 185.7660u | 388.7968m | -0.21814 |
|  | 0.25 | 0.18 | 1.39 | 1.6 | 1.8 | 185.7660u | 388.7968m | -0.21814 |

- DC transfer curve of a CMOS inverter with different pMOS/nMOS beta ratios  assuming the electron mobility is three times of the hole mobility, i.e.,  . Plot the DC transfer curves of the Inverter with seven different ratios:  given in the following table. Find the parameters VIH, VOH, VIL, VOL, NMH, NML related to noise margin for each ratio.

|  | Width Wp (um) | Width Wn (um) | Wp/Wn | VIH (V) | VOH (V) | VIL (V) | VOL (V) | NMH (V) | NML (V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 75 | 2.5 | 30 | 1.30 | 1.70 | 1.10 | 0.05 | 0.4 | 1.05 |
| 5 | 37.5 | 2.5 | 15 | 1.25 | 1.68 | 1.02 | 0.06 | 0.43 | 0.96 |
| 2 | 15 | 2.5 | 6 | 1.15 | 1.66 | 0.88 | 0.08 | 0.51 | 0.80 |
| 1 | 7.5 | 2.5 | 3 | 1.05 | 1.68 | 0.75 | 0.10 | 0.63 | 0.65 |
| 1/2 | 3.75 | 2.5 | 3/2 | 0.93 | 1.71 | 0.62 | 0.11 | 0.78 | 0.51 |
| 1/5 | 1.5 | 2.5 | 3/5 | 0.80 | 1.74 | 0.52 | 0.10 | 0.94 | 0.42 |
| 1/10 | 0.75 | 2.5 | 3/10 | 0.72 | 1.74 | 0.48 | 0.09 | 1.02 | 0.39 |

[figure]：

[75u]:

[37.5u]:

[15u]:

[7.5u]:

[3.75u]:

[1.5u]:

[0.75u]: